Consortium Members

ESA Sea Level CCI

Validation Report: WP2400 Oceanic tidal models

| Reference: | CLS-DOS-NT-12-57 |
| --- | --- |
| Nomenclature: | SLCCI-VR-25 |
| Issue: | 1. 0 |
| Date: | Mar. 12, 12 |

| Chronology Issues: |  |  |  |
| --- | --- | --- | --- |
| Issue: | Date: | Reason for change: | Author |
| 1.0 | 12/03/12 | Initialization | M. Ablain |

| People involved in this issue: |  |  |
| --- | --- | --- |
| Written by (*): | L. Carrere, H. Roinard, M. Ablain | Date + Initials:( visa or ref) |
| Checked by (*): | G Timms (Logica) | Date + Initial:( visa ou ref) |
| Approved by (*): | G Larnicol (CLS) | Date + Initial:( visa ou ref) |
| Application authorized by (*): | ESA | Date + Initial:( visa ou ref) |

*In the opposite box: Last and First name of the person + company if different from CLS

| Index Sheet: |  |
| --- | --- |
| Context: | Baghera tool, project ACT-OCEAN |
| Keywords: | Oceanography, sea level |
| Hyperlink: |  |

| Distribution: |  |  |
| --- | --- | --- |
| Company | Means of distribution | Names |
| CLS | Notification |  |

List of tables and figures

List of tables:

Aucune entrée de table d'illustration n'a été trouvée.

List of figures:

Figure 14: [Diagnosis A01] Standard deviation differences of tide models (GOT4.8 - GOT4.7) for Jason-1	2

Figure 15: [Diagnosis A01] Standard deviation differences of tide models (GOT4.8 – FES 2004) for Envisat	2

Figure 16: [Diagnosis A11] Differences of SLA trends computed with the GOT4.8 model and with  the GOT4.7 model for  Envisat mission (above) and Jason-1 mission (below).	4

Figure 17: [Diagnosis A11] Differences of SLA trends computed with the GOT4.8 model and with  the FES 2004 model for  Envisat mission (above) and Jason-1 mission (below).	5

Figure 18: [Diagnosis ADD2] Differences of amplitudes of 58.74 days signal estimated from SLA corrected from GOT4.8 or GOT4.7, for Jason-1 mission (+/-0.5 cm).	6

Figure 19: [Diagnosis ADD2] Differences of amplitudes of 58.74 days signal estimated from SLA corrected from GOT4.8 or FES 2004, for Jason-1 mission (+/-0.5 cm).	6

Figure 20: [Diagnosis A05] Differences of SSH crossovers  computed with the GOT4.8 model and with the GOT4.7 model for Envisat (left) and Jason-1 (right) missions.	7

Figure 21: [Diagnosis A09] Differences of SLA residual variance computed with the GOT4.8 model and with the GOT4.7 model for Envisat (left) and Jason-1 (right) missions.	8

Figure 22: [Diagnosis A05] Differences of SSH crossovers computed with the GOT4.8 model and with the FES 2004 model for Envisat (left) and Jason-1 (right) missions.	8

Figure 23: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the GOT4.7 model for Envisat mission.	9

Figure 24: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the GOT4.7 model for Jason-1 mission.	9

Figure 25: [Diagnosis ADD1] Differences of residual SLA variance computed with the GOT4.8 model and with the GOT4.7 model for Jason-1 mission (cm²).	10

Figure 26: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the FES 2004 model for Envisat mission.	10

Figure 27: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the FES 2004 model for Jason-1 mission.	11

List of items to be confirmed or to be defined

Lists of TBC:

Aucune entrée de table des matières n'a été trouvée.

Lists of TBD:

Aucune entrée de table des matières n'a été trouvée.

Applicable documents

AD 1 Sea level CCI project Management Plan
CLS-DOS-NT-10-013

Reference documents

RD 1 Manuel du processus Documentation
CLS-DOC

List of Contents

1. Introduction	1

2. Global Mean Sea Level	1

2.1. Long-term evolution	1

2.1.1. Validation diagnoses used	1

2.1.2. GOT4.8 Tide model	1

2.2. Inter-annual signals	1

2.2.1. Validation diagnoses used	1

2.2.2. GOT4.8 Tide model	2

2.3. Annual, semi-annual signals and 60-day signals	3

2.3.1. Validation diagnoses used	3

2.3.2. GOT4.8 Tide model	3

2.3.3. 58.74 days signals	3

3. Regional Mean Sea Level	3

3.1. Long-term evolution	3

3.1.1. Validation diagnoses used	3

3.1.2. GOT4.8 Tide model	3

3.2. 58.74-day signals	5

4. Mesoscale	7

4.1. Validation diagnoses used	7

4.2. Global analyses	7

4.3. Regional analyses	8

5. Conclusions and recommendations	11

Appendix A - Synthesis	12

Appendix B - Definition of the indicator value	14

Appendix C - List of acronyms	15

# Introduction

The main objective of this document is to provide the analysis of the RRDP reports dedicated to the oceanic tidal model in open ocean (WP2400) in order to estimate the best algorithm to compute this correction and improve the sea-level calculation for climate applications.

The following Round Robin Data Packages (RRDP) have been performed for Envisat and Jason-1 where the impact of new GOT4V8 tide model have been analyzed in comparison with GOT4.7 and FES04 models:

- RRDP_WP2400_Tide_GOT4V8_vs_FES04_11-08-25.pdf

- RRDP_WP2400_Tide_GOT4V8_vs_GOT4V7_11-08-25.pdf

GOT4.8 is the last version of the GOT model produced by R. Ray (2011); the difference with GOT4.7 is due to a better processing of the dry tropospheric correction for altimeter data (correcting for S1 and S2 effects).

This document discuss the impact of all new algorithms separating the different climate applications defined in the sea level CCI URD (User Requirement Document) and separating the several temporal scales related with climate applications. A clearly and easily understandable impact indicator has been defined and is described in annex of this document (see Erreur ! Source du renvoi introuvable.).

# Global Mean Sea Level

## Long-term evolution

### Validation diagnoses used

The validation diagnostic of the long-term sea-level evolution (A08-a) allows us to evaluate the impact on the global MSL trend using successively the different tidal model. Their impact is also analyzed separating descending and ascending passes (A08-b): the reduction of the MSL trend differences is a good quality criterion to determine which correction is the best one.

### GOT4.8 Tide model

Using GOT 4.8 instead of GOT4.7 or FES04 models has no impact on global MSL trend estimation.

## Inter-annual signals

### Validation diagnoses used

The monitoring of the differences between both corrections (A01) but also of the variance differences of SLA (A05) may provide information concerning the impact of the studied correction on the global MSL at inter-annual time scales.

### GOT4.8 Tide model

Considering GOT4.8 – GOT4.7 differences, no inter-annual signal is visible on ENVISAT data. Considering J1 data, we observe a diminution of the standard deviation of the difference of both models by 1 mm on the end of the period (from 2009); this variability is not explained.

We also note a slight diminution of the difference GOT4.8 – FES 2004 as a function of time, mostly visible for ENVISAT mission.

Figure 1: [Diagnosis A01] Standard deviation differences of tide models (GOT4.8 - GOT4.7) for Jason-1

Figure 2: [Diagnosis A01] Standard deviation differences of tide models (GOT4.8 – FES 2004) for Envisat

## Annual, semi-annual signals and 60-day signals

### Validation diagnoses used

The periodograms of differences between two different tide models allow us to determine the impact of the studied correction at annual and semi-annual scales (A03). Analyzing the difference of sea-level periodograms (A12), we can describe which correction allows us to reduce the periodic signals. Generally, a reduced annual or semi-annual signal is a good indication of a better correction.

### GOT4.8 Tide model

None significant impact is detected for these periods.

### 58.74 days signals

Diagnostic ADD3 shows the periodograms of the residual SLA corrected from either tide model.

The analysis shows that Jason-1 residuals corrected from GOT4.8 show a lower amplitude for 58.74 days frequency (1.5 mm) than when corrected from GOT4.7 (3 mm).

However, Jason-1 residuals corrected from GOT4.8 still show a higher amplitude for this frequency than when corrected from FES 2004 (0.5 mm).

# Regional Mean Sea Level

## Long-term evolution

### Validation diagnoses used

The validation diagnosis of the regional trend of sea-level differences using successively tidal models (A11a) allows us to evaluate the impact of the different corrections on the local MSL trends. Their impact is also analyzed separating descending and ascending passes (A11b): the reduction of the MSL trend differences is a good quality criterion to determine the best correction.

### GOT4.8 Tide model

Using either GOT4.8 or GOT4.7 model doesn’t have any significant impact on regional MSL trend estimations.

However using GOT4.8 instead of FES 2004 model can have a significant impact on regional MSL in several regions. Concerning Envisat regional MSL, the impact is visible in deep ocean with a stronger impact in Indian ocean and between Australia and New-Zealand, in some shallow water regions and at high latitudes. Concerning Jason-1, there is no impact in deep ocean, but a strong impact (1-2 mm/yr) at high latitudes and in some shallow water regions.

Figure 3: [Diagnosis A11] Differences of SLA trends computed with the GOT4.8 model and with  the GOT4.7 model for  Envisat mission (above) and Jason-1 mission (below).

Figure 4: [Diagnosis A11] Differences of SLA trends computed with the GOT4.8 model and with  the FES 2004 model for  Envisat mission (above) and Jason-1 mission (below).

## 58.74-day signals

Diagnostic ADD2 shows the differences between 58.74 days signal amplitude computed from SLA corrected from either tide model.

Considering the GOT4.8 - GOT4.7 differences, we clearly see the impact of the corrected S2 in GOT4.8. 58.74 days signal differences show a diminution reaching 0.5 cm in deep ocean; they are mostly localized at mid-latitudes.

If comparing GOT4.8 and FES 2004, many differences exist both in deep ocean and in all shallow waters; amplitude of these differences reach more than 0.5 cm.

Figure 5: [Diagnosis ADD2] Differences of amplitudes of 58.74 days signal estimated from SLA corrected from GOT4.8 or GOT4.7, for Jason-1 mission (+/-0.5 cm).

Figure 6: [Diagnosis ADD2] Differences of amplitudes of 58.74 days signal estimated from SLA corrected from GOT4.8 or FES 2004, for Jason-1 mission (+/-0.5 cm).

# Mesoscale

## Validation diagnoses used

Along-track sea-level analyses and differences at crossover points allow us to detect improvements at short temporal scales (< 2months) for mesoscale applications. The most relevant diagnostics performed in RRDP are the monitoring and the map of the variance SSH differences using successively 2 different tidal models.

Diagnostics A05 and A07 display the monitoring and the map of SSH variance differences at crossover points: the reduction of variance indicates a better homogeneity of the sea-level between ascending and descending tracks within a 10-day window.

Diagnostics A09 and ADD1 display the monitoring and the map of SSH variance differences relative to a mean sea surface (MSS): the reduction of variance indicates a better homogeneity with the MSS. Most of the time, it indicates an improvement of the sea-level computation. But note that in few cases, the variance increase can also indicate a systematic error in the MSS due to geographical bias for instance.

## Global analyses

The impact of using GOT4.8 model instead of GOT4.7 is globally weak (between 0.1 and 1.6 cm²) for both missions tested. The impact is half positive and half negative (annual cycle clearly visible on ENVISAT SLA data) with a global mean nearly null.

If considering GOT 4.8 versus FES 2004, we observe a strong positive impact (10-40 cm²) of the new GOT4.8 model as one could expect, because GOT4.8 is very similar to previous version GOT4.7 which already diminished significantly the residual variance of SSH and SLA.

Figure 7: [Diagnosis A05] Differences of SSH crossovers  computed with the GOT4.8 model and with the GOT4.7 model for Envisat (left) and Jason-1 (right) missions.

Figure 8: [Diagnosis A09] Differences of SLA residual variance computed with the GOT4.8 model and with the GOT4.7 model for Envisat (left) and Jason-1 (right) missions.

Figure 9: [Diagnosis A05] Differences of SSH crossovers computed with the GOT4.8 model and with the FES 2004 model for Envisat (left) and Jason-1 (right) missions.

## Regional analyses

The following figures present the impact of the tidal models on SSH crossovers and on SLA residual variance on the global ocean for ENVISAT and Jason-1 missions.

Concerning GOT4.8 vs. GOT4.7 and Jason-1 results, the analysis shows clearly the impact of the S1S2 correction in GOT4.8: improvements are mostly localized between latitudes +/-40 and in some coastal areas (amplitude of 0.25-1 cm), which is coherent with the pattern of the difference between both models. We also notice a degradation of the solution in the Hudson Bay. Analysis on crossovers also shows the improvement of the new solution but not on the same areas due to the time differences (DT) of the crossovers differences which do not always allow observing properly the S2 signal, particularly in the equatorial region. The impact of the new model is very weak when using ENVISAT data due to its sun-synchronous characteristic. However, thanks to more appropriated crossovers DT in some areas, we notice a raise of the variance when using GOT4.8 model in the Hudson bay and in the Norwegian and the Barents seas; this degradation might be explained by errors due to ice cover.

If comparing GOT4.8 vs. FES 2004, we observe a strong local positive impact of the new GOT4.8 model as one could expect, because GOT4.8 is very similar to previous version GOT4.7 which already diminished significantly the residual variance of SSH and SLA. Results for both missions tested show that GOT4.8 allows reducing drastically the residual variance (for along-tracks residuals and crossovers) in shallow waters regions with a variance reduction greater than 25 cm²; both models are equivalent in deep ocean. The analysis on ENVISAT data points out some high latitude regions that are characterized by a rising of the residual variance when using GOT4.8 (Weddel and Ross seas and Arctic ocean north of Bering strait): this degradation is likely explained by errors dues to the ice cover. The impact of the S1S2 correction in the dry tropospheric is not specifically visible here because it is very weak compared to the differences between GOT4.8 and FES 2004 for all other waves.

Figure 10: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the GOT4.7 model for Envisat mission.

Figure 11: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the GOT4.7 model for Jason-1 mission.

Figure 12: [Diagnosis ADD1] Differences of residual SLA variance computed with the GOT4.8 model and with the GOT4.7 model for Jason-1 mission (cm²).

Figure 13: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the FES 2004 model for Envisat mission.

Figure 14: [Diagnosis A07] Differences of SSH crossovers computed with the GOT4.8 model and with the FES 2004 model for Jason-1 mission.

# Conclusions and recommendations

For the global ocean, the GOT4.8 model provides a small variance reduction on global ocean compared to GOT4.7, but a weak degradation is also detected in the Hudson Bay and in the Norwegian and the Barents seas. However, using GOT4.8 allows us to reduce 58.74-days signal estimated on MSL compared to GOT4.7 although this signal remains still stronger than if using FES 2004.

- For Global ocean, we recommend to use GOT4.8 tidal model

Notice that in WP2700 dedicated to high latitudes areas, we studied the impact of 2 others oceanic tide models: DTU10 and TPXO. The analysis of the impact of these both models is performed in the dedicated validation report: SLCCI-ValidationReport_WP2600_HighLatitudes.docx. In this report, we recommend to use the DTU10 tidal model in the Arctic area and northern very high latitudes regions.

- Synthesis

This section synthesizes the impact of the new algorithms dedicated to the SSB correction for the Envisat altimetric mission and separating the different climate applications defined in the sea level CCI URD (User Requirement Document). The impact is also estimated for several temporal scales impacting climate studies for each application.

In order to have a clear view of these potential impacts, the information is summarized in a table (1 table per altimetric missions). An impact indicator clearly and easily comprehensible has been defined with 3 levels: significant impact, low impact, no impact detected. Each level is represented by a different color box.

The choice of a value indicator (significant, low or null) is quite subjective. As it depends on the application (Global MSL, regional MSL, mesoscale…), the rule to classify this impact has been defined in annex of this document (see Appendix B -).

- Definition of the indicator value

In this table, the choice of the indicator value is defined for each climate applications and temporal scales. The thresholds defined here are valid for time series long enough (> 7 years). If time series is too short, the thresholds have to be majored.

| Climate Applications | Temporal Scales | Definition of the indicator value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Significant impact | Low impact | No impact detected |
| Global Mean Sea Level | Long-term evolution (trend) | Trend >0.15 mm/yr | Trend> 0.05 mm/yr | Trend< 0.05 mm/yr |
|  | Inter annual signals (> 1 year) | Amplitude> 0.5 mm | Amplitude> 0.2 mm | Amplitude< 0.2 mm |
|  | Annual and semi-annual Signals | Amplitude> 1 mm | Amplitude> 0.2 mm | Amplitude< 0.2 mm |
| Regional Mean Sea Level | Long-term evolution (trend) | Trend > 0.5 mm/yr | Trend> 0.1 mm/yr | Trend< 0.1 mm/yr |
|  | Annual and semi-annual Signals | Amplitude> 5 mm | Amplitude> 0.5 mm | Amplitude< 0.5 mm |
| Mesoscale | Signals < 2 months | Crossovers Variance differences > 1 cm² | Crossovers Variance differences > 0.2 cm² | Crossovers Variance differences < 0.2 cm² |
| Specific regional areas of main interest for climate studies: |  |  |  |  |
| Coastal areas | Long-term evolution (trend) | Trend > 0.5 mm/yr | Trend> 0.1 mm/yr | Trend< 0.1 mm/yr |
|  | Signals < 2 months | Crossovers Variance differences > 1 cm² | Crossovers Variance differences > 0.2 cm² | Crossovers Variance differences < 0.2 cm² |
| High latitudes | Long-term evolution (trend) | Trend > 0.5 mm/yr | Trend> 0.1 mm/yr | Trend< 0.1 mm/yr |
|  | Signals < 2 months | Crossovers Variance differences > 1 cm² | Crossovers Variance differences > 0.2 cm² | Crossovers Variance differences < 0.2 cm² |

- List of acronyms

| TBC | To be confirmed |
| --- | --- |
| TBD | To be defined |
| AD | Applicable Document |
| RD | Reference Document |